# AVEX Engine cleaning guide.

Review the safety section at the end of this guide before performing engine cleaning.

Use of appropriate PPE is required when performing this job.

- Gather the following materials. PPE: painting suit, respirator, ear protection, face shield, and gloves. Engine washing sprayer, 1 gallon can of mineral spirits, thin painting plastic, blue painters tape and acquire red rags and white rags.

- Protect aircraft from chemical overspray by adding plastic covering to windshield and side windows, cover nose tire with trash bag or plastic. Place exhaust stacks covers on, place propeller blade covers on. If not already accomplished cover inlets to air conditioning system. Place absorbent mats on ground for length of the engine. Move air hose to aircraft.

- Verify that the aircraft does not have a battery installed or ground power applied. Remove GPU connection if installed. (Older aircraft if powered had a shunt near the GPU receptacle which will heat up and cause a fire hazard when combined with the atomized mineral spirits). FIRE INFORMATION: Mineral spirits is used in some applications as starting fluid and is combustible and a fire hazard.

- Begin cleaning by wiping away excess grease from nose gear steering linkages and also nose gear up lock springs. Using red towels.

- Using the engine cleaning wand and remove remaining grease from nose gear steering and springs and attempt to dissolve any grease clumps. Then spray bottom of engine (Accessory gearbox) nose gear doors and bottom of cowling..

- Clean the tops of the cowlings, including moving latches back and forth while spraying to clean mechanism.

- Clean cowl and use acid brush if necessary to remove dirt around rivet lines. Do not forget to clean under the spine that connects the two cowlings

- Starting from the LH side clean the top of the engine to the bottom then move forward towards the propeller.

- Do not spray closely the propeller to engine flange or the propeller blade roots to prevent blowing out the o-rings.

- Clean propeller hub with mineral spirit spray but from a distance of at least 12 inches.

- Clean from top to bottom RH side of engine and then pay special attention to the engine control linkages at the FCU and CAM box.

- Clean engine fire wall and accessories.

- Clean engine tubular mount thoroughly including wiring and hoses adjacent.

- Clean bottom of cowling and nose landing gear.

- Re-clean nose gear steering linkages to catch anything you missed

- Clean bottom of engine and nose gear doors again

- Re-wet the engine by spraying

- Wipe off entire engine with white terry towels and simple green.

SAFTEY: Information from the Mineral Spirits safety data sheet (SDS):

INGESTION (SWALLOWING): May be harmful if swallowed. May cause throat irritation, nausea, vomiting, diarrhea and central nervous system effects as noted under

INHALATION (BREATHING). Aspiration hazard: breathing product into the lungs during ingestion or vomiting may cause lung injury and possible death.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE: Individuals with pre-existing respiratory tract (nose, throat, and lungs), central nervous system, eye, and/or skin disorders may have increased susceptibility to the effects of exposure.

CHRONIC: Prolonged or repeated inhalation may cause toxic effects as noted under INHALATION (BREATHING). Prolonged or repeated eye contact may cause inflammation of the membrane lining the eyelids and covering the eyeball (conjunctivitis). Prolonged or repeated skin contact may cause drying, cracking, redness, itching, and/or swelling (dermatitis)

Fire information: Paint Thinner (mineral spirits), Paint thinner or mineral spirits is a complex petroleum distillate; flash point range is listed as 104 degrees F. Ignition temperature 473 degrees F, toxicity (3). A category of clear, combustible liquid having petroleum type odor. Mineral spirits are midrange petroleum distillates which is present in many paint thinners, oil base stains, dry cleaning solvents, and some brands of charcoal starter fluids. .